COA ASSIGNMENT 01

Aaditya Yuvaraj Hande

(202401040028)

Roll No. 17

Batch : A1

1. A: Addition of 2 Numbers

# ALP:

MOV #7, R00

MOV #14, R01 ADD R00, R01 MOV R01, R02 HLT

Addition of 2 Numbers, Taking Input

# ALP:

IN 0, R00

SUB #48, R00

IN 0, RO1

SUB #48, R01

ADD R00, R01

CMP #9, R01

JLE 45

ADD #7, R01

ADD #48, R01	; logical address – 45

OUT R01, 1

HLT

Input:

First Number: 7

Second Number: 8

Addition = 15

Output:

= 15+7+48

= 70 ASCII Value -> F

Result Output = F

1. B: Average of Two Numbers

# ALP:

MOV #7, R00

MOV #7, R01

ADD R00, R01

DIV #2, R01

MOV R01, R02

STB R02, 0

HLT

1. B: Average of Three Numbers

# ALP:

MOV #101, R00

MOV #102, R01

Mov #100, R02

ADD R00, R01

ADD R01, R02

DIV #3, R02

MOV R02, R03 STB R03, 0 HLT